# Raspberry Pi 5 to Arduino Mega Communications (Serial)

Commands being sent from Rpi to Arduino:

- “PAUSE”

- When [Pause] button is pressed on touchscreen during session

- Same button is now changed to [Play].

  - “RESUME”

- When [Play] button is pressed on touchscreen during session

- Same button is now changed to [Pause].

- “STOP”

- When [Stop] button is pressed on touchscreen during session

- Function: To stop the timer & return to main menu.

- “START”

- When [Start] button is pressed on touchscreen after settings selection

- Function: To start countdown of sessions & notify Arduino that the timer has started.

- “TIMER_END”

- When countdown on screen reaches 0:00, signifying that the session has ended.

- Function: To notify Arduino that the countdown timer has ended and stops all operation.

# Function to send global settings to Arduino

def send_settings(SOUTHPAW_MODE, FOOTWORK_MODE, ROUNDS, SPEED, TIMER, LEVEL, REST, COUNTER, BOXING_MODE, SELECTED_PUNCHES):

settings = {

"SOUTHPAW_MODE": SOUTHPAW_MODE,

"FOOTWORK_MODE": FOOTWORK_MODE,

"ROUNDS": ROUNDS,

"SPEED": SPEED,

"TIMER": TIMER,

"LEVEL": LEVEL,

"REST": REST,

"COUNTER": COUNTER,

"BOXING_MODE": BOXING_MODE,

"SELECTED_PUNCHES": ','.join(SELECTED_PUNCHES)

# Convert list to a comma-separated string

}

for key, value in settings.items():

message = f"{key}:{value}\n"

ser.write(message.encode())  	# Send settings to Arduino

time.sleep(0.1)  			# Add a small delay between messages

# Function to send control signals to Arduino

def send_control_signal(signal):

ser.write(f"{signal}\n".encode())  # Send signalS to Arduino (START, PAUSE,

RESUME, STOP, TIMER_END)

# Function to start the boxing interface

def start_interface(root, SOUTHPAW_MODE, FOOTWORK_MODE, ROUNDS, SPEED, TIMER, LEVEL, REST, COUNTER, BOXING_MODE, SELECTED_PUNCHES):

# Send settings before starting

send_settings(SOUTHPAW_MODE, FOOTWORK_MODE, ROUNDS, SPEED, TIMER, LEVEL, REST, COUNTER, BOXING_MODE, SELECTED_PUNCHES)  # Send settings to Arduino

# Notify Arduino that the timer has started

send_control_signal("START")

- "SOUTHPAW_MODE": SOUTHPAW_MODE,

- OFF/ON

- "FOOTWORK_MODE": FOOTWORK_MODE,

- OFF/ON

- "ROUNDS": ROUNDS,

- 1 -12

- "SPEED": SPEED,

- 25

- 50

- 75

- 100

- "TIMER": TIMER,

- 1:00

- 1:30

- 2:00

- 3:00

- "LEVEL": LEVEL,

- 1

- 2

- 3

- "REST": REST,

- 0:10

- 0:30

- 0:45

- 1:00

- "COUNTER": COUNTER,

- OFF/ON

- "BOXING_MODE": BOXING_MODE,

- SPAR/COMBO

- "SELECTED_PUNCHES": ','.join(SELECTED_PUNCHES)

- 1

- 2

- 3

- 4

- 5

- 6

- L-Pivot

- R-Pivot

17/10/2024 4:05 am:

Data Received (displayed on LCD) : SOUTHPAW_MODE, FOOTWORK_MODE, ROUNDS, SPEED, TIMER, LEVEL, REST, “TIMER_END”

Data not received : COUNTER, BOXING_MODE, SELECTED_PUNCHES

START_ROUND  after rest time is over for next round to start, or

STOP  only when the rounds are ended

RELAX  when rest time has started after the round has ended

The function send_control_signal() in this code is responsible for sending specific control signals to the Arduino (or another control device) to manage the robot's actions during different stages of a boxing session. It is called at specific points within the round_timer_interface() function to indicate state changes, like starting a round, entering a rest period, or stopping the session altogether. Here’s how send_control_signal() is used within the round_timer_interface() function:

Context for Each Call to send_control_signal()

- Starting a Round:

  - send_control_signal("START_ROUND") is called initially when the interface is set up and whenever a new round starts after a rest period.

  - It signals the Arduino that a new round has started, prompting it to begin actuating the motors or carrying out other active tasks for the boxing round.

- Entering Rest Mode:

  - When a round's timer expires, send_control_signal("RELAX") is sent, triggering a rest period.

  - This likely tells the Arduino to stop motor actuation or reduce activity, allowing the user a break before the next round.

- Completing All Rounds:

  - Once all rounds are completed, send_control_signal("STOP") is sent.

  - This signals the Arduino to halt all activities related to the boxing session, ensuring the robot doesn’t perform any further movements.

Breakdown of the round_timer_interface() Function

- Timer and Round Management:

  - This function keeps track of the number of rounds (ROUNDS), the time per round (TIMER), and the rest period between rounds (REST).

  - It displays the current round and countdown timer on the screen.

- update_timer() Function:

  - Runs the main round timer. Each second, it decreases timer_seconds and, if necessary, timer_minutes.

  - When the round time is up, it increments current_round and, if all rounds are done, sends the STOP signal.

  - If rounds remain, it sends the RELAX signal, initiating the rest period through start_rest_timer().

- start_rest_timer() Function:

  - Manages the countdown for the rest period between rounds.

  - Sends the START_ROUND signal when the rest period ends, kicking off the next round and resetting the round timer.

In summary, send_control_signal() is used to maintain communication with the Arduino to manage the robot’s behavior at the start of each round, during rest periods, and at the end of all rounds, ensuring a smooth transition between the different phases of the workout.