To size the batteries required for operation of the mecanum wheels, we look at the motor parameters. The target specification of a BoxBunny training session is 30 minutes, so the battery must be able to power the 4 motors for at least 30 minutes.

24V 120W rating

Runs efficiently and safely at 24V. Delivers up to 120W of mechanical power under optimal conditions (rated torque of 2.5Nm at rated speed of 350rpm).

No-load current mA: ≦ 600

Current that the motor draws when it’s running without any load.

Rated current A: ≦ 2.7

Current that the motor draws when it is operating under its rated load (rated torque of 2.5Nm at rated speed of 350rpm). This is the typical current the motor will draw when it is working as intended.

Stall current A: ≦ 21

Maximum current that the motor draws when it is stalled (i.e. not moving, under full load or locked). Must have circuit protection (e.g. fuse, current limiter) to prevent damage in case of a stall.

Target specification of a BoxBunny training session is 30 minutes, so the battery must be able to power the 4 motors for at least 30 minutes.

With the above specifications, the selected power supply should be

- Rated 24V

- Minimum 2.7A

- Contingencies for momentary peaks up to 21A

- Capacity for 4 motors for at least 30 minutes

  - Rated current of 2.7A

  - For 4 motors, at least 30 minutes, we need battery capacity of at least 5.4Ah

Other considerations:

- Battery life

  - Actual running time will decrease with repeated use

- Safety

  - Motor has a stall current of 21A, need to ensure battery is protected against high currents

Proposed battery sizing

24V 10Ah 18650 7S4P Lithium ion battery (SGD234.53)

https://www.battery.com.sg/product/24v-10ah-18650-7s4p-lithium-ion-battery

- Able to power 4 motors for roughly 55 minutes

- Comes with protection circuit

- L15 x W8.5 x H7 cm

29.4V 7 cells Li-ion battery charger (SGD40.18)

https://www.battery.com.sg/product/294v-7-cells-li-ion-battery-charger

- Terminal: 5.5 x 2.5 DC plug

- L 108 mm x W 48 mm x H 28mm