# Design Considerations for Robot Structure Mechanical

The primary procurement undertaken during this project phase was the Screw Jack, coupled with its compatible motor, which together serve as the driving mechanism for adjusting the robot’s height. There were two design segments that had to be heavily considered for the overall stability of the robot: Telescopic Tubes, and Base Support.

## Technical Information of Driving Mechanism

The height-adjustment mechanism of the robot is implemented using a motorised screw jack driven by a three-phase AC induction motor. The screw jack provides vertical linear motion via a travelling nut, while the motor supplies the rotational input. The system is designed for controlled, bidirectional vertical motion (up/down) with end-of-travel protection using limit switches. Table 1 shows the technical specifications for the screw jack and motor.

Table 1: Screw Jack Assembly Specifications

| Component | Parameter | Specification |
| --- | --- | --- |
| Screw Jack | Model | HK2T |
|  | Rated lifting capacity (selected) | 300 kg (design requirement) |
|  | Structure type | Travelling nut type |
|  | Nut material | Copper (brass) nut |
|  | Stroke length | 400 mm |
|  | Gear ratio | 1:10 (selected configuration) |
|  | Hoist speed | ~0.70–0.75 m/min |
|  | Estimated travel time (400 mm) | ~32–34 s |
|  | Installation | Vertical mounting, guided by external wear-pad sleeves |
| Motor | Motor type | Three-phase AC induction motor |
|  | Mounting type | B5 flange mount |
|  | Rated power | 0.37 kW |
|  | Rated speed | 1400 r/min (rpm) |
|  | Rated voltage | 380 V / 220 V (dual voltage) |
|  | Rated current | 1.12 A (380 V, Y) / 1.92 A (220 V, Δ) |
|  | Rated frequency | 50 Hz |
|  | Connection | Y (380 V) / Δ (220 V) |
|  | Protection class | IP54 |
|  | Insulation class | Class F |
|  | Motor mass | 6.9 kg |

## Control System Design

Electrical power is supplied from a standard 230 V single-phase wall outlet. The incoming power is first routed through basic protective devices (MCB/RCD/fuse) and a main switch to ensure overcurrent and fault protection. This should be split into two branches:

- Branch A (high power): VFD/contactor system  screw jack motor

- Branch B (low voltage): 230V  transformer to lower voltages for other robot components

This keeps motor noise from killing the other electrical components.

A Variable Frequency Drive (VFD) is used to:

- Convert single-phase AC input into three-phase AC output,

- Supply the appropriate voltage and frequency required by the motor,

- Enable forward and reverse motor operation for bidirectional screw jack travel.

Although speed control is not required for this application, the VFD functions as a phase converter and motor starter, enabling compatibility between wall-plug power and a three-phase induction motor.

The VFD drives a 0.37 kW, three-phase AC induction motor mounted via a B5 flange directly to the screw jack input. The motor operates at approximately 1400 r/min under rated conditions.

Rotational motion from the motor is transmitted through the screw jack’s internal reduction gearbox (ratio 1:10), converting high-speed, low-torque input into low-speed, high-torque output suitable for vertical lifting.

The screw jack employs a travelling nut mechanism, which translates rotational input into vertical linear motion with a stroke of 400 mm. The travelling nut is mechanically connected to the robot’s upper structure.

To prevent lateral loading on the screw and nut, vertical motion is constrained using a guided column system consisting of wear pads or guide sleeves. These guides absorb lateral forces arising from user interaction (e.g., punching impacts), ensuring that the screw jack primarily experiences axial loads.

User Interface

Bidirectional motion (UP/DOWN) is commanded electrically via either:

- A reversing cam switch (UP–OFF–DOWN), or

- Two momentary push buttons (UP / DOWN).

End-of-travel Protection

Two limit switches are installed at the upper and lower ends of travel:

- The top limit switch interrupts upward motion at maximum height,

- The bottom limit switch interrupts downward motion at minimum height.

These limit switches are wired into the control circuit to provide automatic stopping and prevent mechanical over-travel, ensuring safe operation of the height-adjustment mechanism.

The height-adjustment actuator is controlled using a single-phase-input variable frequency drive (VFD) configured for external terminal control. Forward and reverse motion are commanded via dedicated digital inputs (M1 and M2) using momentary UP and DOWN pushbuttons. End-of-travel protection is implemented using upper and lower limit switches wired in series with the corresponding direction command. Although normally open limit switches are used in the current prototype, the control logic ensures that motion is inhibited at the travel limits while still allowing reversal away from the end position. The VFD eliminates the need for external reversing contactors and provides integrated motor protection and soft-start capability.

## #1: Telescopic Guided Column System

## #2: Base Support

Building a bolt-together extrusion tower + thick top adapter plate + dedicated motor support shelf + one shear panel

This keeps everything off-the-shelf, no welding, easy to dismantle, and it directly addresses the 3 real failure modes: racking/twist, vibration loosening, and load path into the base.

Why this is the best “value” option

- Axial load: The jack rating (200 kg on travelling nut) is not the limiter; your structure is. A braced extrusion tower with a thick top plate will comfortably handle your current and near-future axial loads.

- Motor vibration: A motor shelf tied to the frame prevents the motor mass from cantilevering on the jack input, which is what causes loosening and bearing wear.

- Indirect punch impacts: Those show up as moments and lateral shocks, not pure axial. The shear panel (or X-brace) is what prevents the whole pedestal from “breathing” and walking loose over time.

- Modular: You can unbolt the whole pedestal as one module later when footwork comes in.

Undertaken a very common engineering path:

- Started from a simple mechanism selection of Hand Crank Screw Jack.

- Discovered integration constraints with the standard screw up/down movement due to its overall height of 400mm stroke both below and above its mounting plate.

- Iterated to a better form that includes a travelling nut with mounting holes.

- Fixed its usability as a user to use motorisation instead of manual hand crank.

Mistake made:

- Should have had tighter requirements with a system-level selection pass, around the speed, user experience, power source and architecture, in order to avoid being cornered into a 3-phase induction motor.

What I should have done:

- Lock requirements before browsing parts

  - Functional

    - Stroke Length: 400mm

    - Height envelope at Min & Max: Max retracted height allowed (can only be determined during designing of the whole robot)

    - Required height-change time (UX): e.g. ≤10 – 15s ideal, ≤30s acceptable

    - Hold position safely at any height (self-locking or brake)

  - Loads

    - Design vertical load:

𝐹𝑑 = robot mass × g × safety factor = 100kg × 9.81 × 1.5 = 1,471.5 N ≈ 1.5 kN

This would include: static weight + dynamic effects (punch vibrations, accidental pushes, off-axis moments).

    - Side-load/moment tolerance: how much the column must tolerate without binding.

  - Power + Integration

    - Available power: do you have 24 V battery on board already? (BoxBunny does) If yes, prefer 24 V DC gearmotor/actuator over 220 V 3-phase.

    - Noise, cost, safety, wiring complexity, control simplicity.

- Choosing a screw jack depends on these parameters

  - Lead (mm/rev) to hit your target height-change time without insane RPM.

  - Efficiency (worm jacks are typically low), to size motor properly.

  - Duty cycle (demo use can overheat small motors).

  - Guiding so the jack doesn’t see side load / bending moment (jack = lifter, not a guide).