# Selection of Transport Guide Wheels

## Tilted Hand-Truck Handling (166.32 kg Robot)

### Design Objective

Select a wheel set to transport a 166,320 g (166.32 kg) robot in a tilted “hand-trolley/hand-truck” mode, including traversal of thresholds and ramps, while minimising risk of wheel, axle, and mount failure and reducing shock transmission into the robot structure.

### Load Case and Key Definitions

Robot mass: m=166.32 kg

Robot weight: W=mg≈166.32×9.81≈1632 N

In a tilted hand-truck configuration, the robot is supported primarily by two wheels, so the wheels experience:

- High steady load (often close to the full robot weight split between two wheels),

- Dynamic load spikes when rolling over obstacles (threshold lips, joints, ramp transitions),

- Uneven load sharing during steering/turning and when one wheel hits a bump first.

Wheel/caster manufacturers distinguish dynamic load capacity (moving, obstacle-affected) from static load capacity (stationary), with dynamic being the relevant rating for transport. Blickle+1

### Required Wheel Rating (Engineering Sizing)

A common sizing approach is to compute required load per wheel and apply a safety factor that accounts for speed, obstacle height relative to wheel diameter, and unevenness. MISUMI+1

Nominal load per wheel (even split, no shock):

m2=166.322=83.16 kg per wheel

Recommended safety factor for thresholds/ramps (dynamic service):

Use S = 1.5 to 2.0 to cover obstacle shocks + imperfect load distribution (turning, one-wheel obstacle contacts first). (This is consistent with industry guidance to apply safety margins rather than dividing by wheel count “as-is”.) MISUMI+2Caster Concepts+2

Dynamic rating target per wheel:

T=83.16×S⇒S=1.5:T≈125 kg/wheelS=2.0:T≈166 kg/wheel
Design requirement (recommended):

- ≥ 170 kg dynamic load rating per wheel (preferred for frequent thresholds)

- ≥ 125 kg dynamic load rating per wheel (minimum for milder obstacles / careful handling)

### Suitability of Two 6-inch Wheelchair Wheels

For 166 kg plus threshold/ramps, two 6" wheelchair wheels are generally not ideal as “guide wheels” in a tilted hand-truck configuration because:

- Dynamic load governs, not static: moving over obstacles introduces stresses that reduce allowable load versus static ratings. Blickle+1

- Small diameter increases obstacle severity: the obstacle height-to-wheel diameter ratio increases shock and required safety factor. MISUMI+1

- System weak link is often the mount/axle/fork, not the tire itself: wheelchair caster assemblies can be robust, but many “replacement wheel” products don’t provide clear dynamic ratings for repeated threshold impacts.

Conclusion: For this load and terrain, 6" wheelchair wheels are a higher-risk choice unless you have verified dynamic per-wheel ratings in the 125–170 kg range and the axle/fork/mount are designed for bending and side loads.

### Recommended Wheel Solution: Hand-Truck Style Pneumatic (or Flat-Free) Wheels

For thresholds/ramps, hand-truck wheels provide better rollover and shock isolation. Typical published ratings for common hand-truck wheels include:

- 8" pneumatic: ~250 lb (113 kg) per wheel → below your recommended dynamic target. Access Casters+1

- 10" pneumatic: ~350 lb (159 kg) per wheel → borderline (OK only if obstacles are mild + careful handling). Access Casters

- 10" pneumatic: ~400 lb (180 kg) per wheel → matches the recommended target. Amazon Singapore+1

- 10" flat-free options: 600 lb capacity are available → strong safety margin for harsh thresholds. Service Caster

- Some wheel specs explicitly list dynamic vs static (e.g., 400 lb dynamic / 450 lb static), which is exactly what you want to compare against your requirement. eur.vevor.com

Selection recommendation for your robot:

- Preferred: 10" wheels rated ≥ 400 lb (≈180 kg) dynamic per wheel

- Best-margin: 10" “flat-free/no-air” wheels rated ≥ 600 lb per wheel if thresholds are frequent or harsh Service Caster+1

### Integration Notes (so the rating is actually achieved)

To ensure the wheel rating is meaningful in your build:

- Use a through-axle with proper bearing support (many hand-truck wheels commonly use 5/8" bore bearings, sometimes 3/4"). Access Casters+1

- Design the wheel bracket/fork to resist bending moments from tilt + steering side loads (this is often the failure point).

- Add retention features: locking nuts, cotter pins, or axle collars; and spacers so the wheel doesn’t rub on the bracket.

- Add a strap/constraint to keep the robot from shifting on the trolley during bump events.

### Design Decision Summary

- Two 6" wheelchair wheels: Not recommended for 166 kg with thresholds/ramps unless you can verify high dynamic rating and robust mounting.

- 10" hand-truck pneumatic/flat-free wheels: Recommended, with ≥ 180 kg dynamic per wheel (400 lb) as the main target; 600 lb per wheel if you want a large safety margin. Preston Hardware+2Service Caster+2