# Strike-zone Height Sizing

Based on pilot measurements from four members of various skill level, the robot’s strike-zone geometry was sized using a ratio-based normalisation approach. For each participant of height H, the preferred strike heights for the head and torso were recorded (hhead,htorso) and converted into dimensionless ratios rhead=hhead/Hand rtorso=htorso/H. This method allows the preferred striking locations to be scaled to different user statures, while preserving body proportions. Across the four samples, the mean ratios were found to be rhead≈0.921and rtorso≈0.711, which were used as baseline targets for defining the head and torso strike-zone locations over the intended user range.

The robot’s overall height adjustment is defined from the ground to the top of the robot head, and is constrained to 150–190 cm, corresponding to the screw-jack stroke of 400 mm. As the lift mechanism translates the head and torso targets together (single-DOF motion), the vertical separation between the head and torso contact points is fixed by design. From the measured data, the head–torso centre spacing for each participant is si=hhead,i-htorso,i. This spacing was observed to vary between 28–40 cm, with an overall mean of approximately 34 cm. Therefore, the robot’s head and torso strike-zone centres were finalised to have a fixed vertical separation of ~34 cm, which represents a reasonable average proportion while staying within the empirical range of human preferences measured in the pilot group.

Finally, because the head and torso targets move together (no independent adjustment), the design accounts for inter-user variability by sizing the strike-zone pads to be sufficiently tall rather than introducing extra adjustment features. In practice, this means the pads are designed as “zones” (not single points), so that users of different proportions can still experience a realistic and hittable target even when their individual preferred head/torso landmarks differ slightly from the robot’s fixed geometry. This approach maintains mechanical simplicity (single screw-jack height adjustment) while ensuring usability across the intended 150–190 cm operating range.

Predicted strike heights at 150 cm and 190 cm (by participant ratios)

| Participant ratio used | Head @ 150 (cm) | Torso @ 150 (cm) | Head @ 190 (cm) | Torso @ 190 (cm) |
| --- | --- | --- | --- | --- |
| Girl 1 | 129.0 | 92.4 | 163.4 | 117.0 |
| Girl 2 | 137.3 | 114.8 | 174.0 | 145.4 |
| Guy 1 | 143.2 | 109.3 | 181.4 | 138.5 |
| Guy 2 | 143.1 | 110.1 | 181.2 | 139.5 |

Design Envelope Summary (min/mean/max across 4 samples)

| Target | @ 150 cm user (min/mean/max) | @ 190 cm user (min/mean/max) |
| --- | --- | --- |
| Head strike height | 129.0 / 138.1 / 143.2 | 163.4 / 175.0 / 181.4 |
| Torso strike height | 92.4 / 106.6 / 114.8 | 117.0 / 135.1 / 145.4 |

- At 150 cm: head 138.1 cm, torso 106.6 cm

- At 190 cm: head 175.0 cm, torso 135.1 cm

- 150cm: Head-torso diff = 138.1-109.6=28.5 cm

- 190cm: Head-torso diff = 175-135.1=39.9 cm

- OVERALL: Head-Torso Center Length = 28 - 40 cm (avg = 34cm)