# System Torque Requirements

The determination of the required torque for accurate and precise rotation requires the rotation system to be characterized first. The pool noodle, our target object of rotation, is firstly characterized based on the existing sample to the desired length. Other parameters like rotation speed and acceleration are accounted for based on our testing and target demographics. The static and dynamic torque are then calculated. Lastly, further suggestions are made, and the final adjusted torque requirements are calculated.

## Arm System Characteristics

### Pool Noodle

The pool noodle, our target object of rotation, is firstly characterized based on the existing sample. We first determine the density of the current pool noodle base on it’s dimensions and weight.

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Length | L | 580 | mm |
| Diameter | D | 60 | mm |
| Mass | M | 49.25 | g |
| Calculated Density | Ρmat | 30 | kg/m3 |

The specification for the new pool noodle is based on the desired reach of the robot.

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Length | L | 900 | mm |
| Diameter | D | 60 | mm |
| Density | Ρmat | 30 | kg/m3 |
| Calculated Mass | M | 76.3 | g |

### 1.1.2 Angular Speed and RPM

Our target matrix is to be able to match the striking tempo of a coach during a training session. We have recreated a series of five 90° strikes and recorded the total duration of these strikes.

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Strike Angle | ф strike | 90 | degree |
| Number of Strikes | n | 5 | Unitless |
| Duration | Tstrikes | 2.62 | s |
| Calculated Angular Speed | ω | 6.04 | rad/s |
| Rotation Per Minute | RPM | 57.7 | rotation/min |

### 1.1.3 Acceleration

The time it takes for the arms to accelerate to the desired speed is crucial to determining the peak torque. A faster acceleration will result in a larger peak torque. To accommodate the users, we base the acceleration time on the human reaction time to visual stimulus, providing the users sufficient time to react. We’ve found that "The average human reaction time is typically 0.25 seconds to a visual stimulus" (Crossley, 2021).

Crossley, J. (2021). Functional exercise and rehabilitation: The neuroscience of movement, pain and performance (1st ed.). Routledge. https://doi.org/10.4324/9781315156842

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Time to full speed | t | 0.25 | s |
| Angular Speed | ω | 6.04 | rad/s |
| Calculated Acceleration | α | 24.2 | rad/s2 |

## Arm System Torque Requirements

### Static Torque

For rotation in a vertical plane, the maximum static torque happens as the noodle passes through the horizontal position (ф = 90). At this point, the gravitational torque is at its maximum.

τg,max=M⋅g⋅L2=0.0763 kg⋅9.81 m/s2⋅0.9 m2=0.337 N⋅m

### Acceleration Torque

To determine the acceleration torque required, we first model the pool noodle as a uniform rod to calculate its inertia.

Iend = 13ML2

Iend = 130.0763 kg0.9 m2
= 0.0206kg∙m2

The acceleration torque is the required to produce the angular acceleration α.

τα=Iend⋅α=0.0206 kg⋅m2⋅24.2 rad/s2=0.50 N⋅m

### Drag Torque

During the rotation of the pool noodle, it also experiences aerodynamic drag which contributes an opposite torque. To maintain a constant rotation, the motor must apply a continuous torque to oppose the opposing drag. The drag force is determined by the following equation:

Fd=12CdρairAυ2

Where Cd is the drag coefficient, and ρair is the density of air. To calculate the resultant torque because of drag, we must consider two other factors. Firstly, the velocity of the rotating pool noodle is not a constant but varies with length from the pivoting point. Secondly, the resultant torque must account for the increasing distance across the pool noodle while still accounting for the increase in velocity. To derive the appropriate equation, we first consider the infinitesimal force along the pool noodle as a function of it’s length.

Velocity υr=rω

dFd=12CdρairD⋅drrω2=12CdρairDω2r2dr

This force acts at a distance r from the pivot, so the infinitesimal torque from drag is:

dτdrag=r⋅dFd=r12CdρairDω2r2dr=12CdρairDω2r3dr

If we integrate from the pivot to the total length of the pool noodle, we can determine the equation to calculate the drag torque.

τdrag=0L12CdρairDω2r3dr

τdrag=18CdρairDω2L4

The coefficient of drag is also not a constant and is dependent on the Reynolds number. We account for the highest Reynolds number which occurs at the tip speed, assuming that there is no wind present.

Re=υDνair

Where νair is the kinematic viscosity of air. https://borgoltz.aoe.vt.edu/aoe3054/manual/expt3/index.html

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Diameter | D | 60 | mm |
| Calculated Angular Speed | ω | 6.04 | rad/s |
| Calculated Tip Speed | u | 5.43 | m/s |
| Dynamic Viscosity of Air | Νair(15°) | 1.81 x 10-5 | m2/s |
| Air Density | Ρair(15°) | 1.225 | kg/m3 |
| Reynolds Number | Re | 1.8 x 104 | unitless |
| Drag Coeff | Cd | 1.2 | unitless |
| Drag Torque | τdrag | 0.215 | N∙m |

### Peak vs. Continuous Torque Requirements

Based on the calculations, the system has two distinct torque requirements.

Peak torque is the maximum, intermittent torque required by the system. It occurs only during the 0.25-second acceleration phase while the arm is simultaneously lifting against the maximum gravitational load (at the horizontal position).

τpeak=τα+τg,max

τpeak=0.50 N⋅m+0.337 N⋅m=0.837 N⋅m

Continuous torque is the torque required to maintain the arm's operation after acceleration is complete. This load is highest when the arm is moving through the horizontal position at full speed, where the motor must fight both gravity and aerodynamic drag.

τcontinuous=τg,max+τdrag

τcontinuous=0.337 N⋅m+0.215 N⋅m=0.552N⋅m

### Final Arm System Specifications

It is standard engineering practice to apply a safety margin to account for modelling inaccuracies, unforeseen friction, variations in material properties, and voltage fluctuations. A safety factor of 1.5 is applied to the required torque.

τpeak×1.5=0.837 N⋅m×1.5=1.26N⋅m

τcontinuous×1.5=0.552N⋅m×1.5=0.828N⋅m

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Peak Torque | Τpeak | 1.26 | N∙m |
| Continuous Torque | Tcontinuous | 0.828 | N∙m |
| Rotation Per Minute | RPM | 57.7 | rotation/minute |

It is often impractical to find a motor that can directly produce the required torque at the desired low speed. The solution is to use a geared system for mechanical advantage. A gear train can decrease the output speed while proportionally increasing the output torque.

## Future Works for Arm System

After presenting a first-principles methodology for determining the torque required to rotate a pool noodle, modelled as a rod, about one of its endpoints, the system requirements will guide us in our subsequent component selection. For example, the required torque can be further reduced if we opt for a hollowed pool noodle, this selection will reduce the weight and thus both the static and acceleration torque.

Once the appropriate arm system has been selected, we will adopt the following methodology for deciding the appropriate drive gear train and motor.

- Prioritize Dual Torque Criteria: The primary selection criterion is a motor-gearbox combination that provides:

  - A rated continuous torque of approximately 0.828 N∙m

  - A peak output torque of approximately 1.26 N∙m

- Utilize a Geared Motor: A high-speed, low-torque DC motor coupled with a reduction gearbox would allow us to generate the required torque while still achieving the desired speed.

- Validate Speed and Control: Ensure that the chosen motor and gear ratio combination can achieve the target speed of 60 RPM.

## Body System Characteristics

### System Components

As body is made up of various key subsystems, the body mass is derived based on their sum.

| Subsystem | Mass | Units |
| --- | --- | --- |
| Height Adjustment - Lifting Screw Jack | 10 | kg |
| Linear Motion - Sliding Table Linear Rail | 20 | kg |
| Arm Gear System | 2 | kg |
| Pool Noodle | 76.3 | g |
| Head | 2 | kg |
| Torso | - |  |
| Total Mass | 200 | kg |

### Acceleration

The time it takes for the arms to accelerate to the desired speed is crucial to determining the peak torque. A faster acceleration will result in a larger peak torque. To accommodate the users, we base the acceleration time on the human reaction time to visual stimulus, providing the users sufficient time to react. We’ve found that "The average human reaction time is typically 0.25 seconds to a visual stimulus" (Crossley, 2021).

Crossley, J. (2021). Functional exercise and rehabilitation: The neuroscience of movement, pain and performance (1st ed.). Routledge. https://doi.org/10.4324/9781315156842

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Time to full speed | t | 0.25 | s |
| Angular Speed | ω | 2.62 (150°/s) | rad/s |
| Calculated Acceleration | α | 10.5 | rad/s2 |

## Body System Torque Requirements

### Acceleration Torque

To simplify our calculations for our initial selection of components, the inertia of the body will be based on solid cylinder.

| Parameter | Symbol | Value | Units |
| --- | --- | --- | --- |
| Total Mass | M | 200 | Kg |
| Diameter | D | 40 | cm |

Ibody = 12M(D2)2

Ibody = 0.238kg∙m2

For the desired acceleration, the resultant acceleration torque is calculated.

τα=Ibody⋅α=0.238 kg⋅m2⋅10.5 rad/s2=2.50 N⋅m

### Final Body System Specifications

It is standard engineering practice to apply a safety margin. A safety factor of 1.5 is applied to the required torque.

τbody×1.5=2.50 N⋅m×1.5=3.75 N⋅m

While the geometry of the body is overly simplified in these calculations, it aids in the initial phase of component selection for testing.

## Future Works for Body System