# Screw Jack Motorised Tech Spec

The primary procurement undertaken during this project phase was the Screw Jack, coupled with its compatible motor, which together serve as the driving mechanism for adjusting the robot’s height. There were two design segments that had to be heavily considered for the overall stability of the robot: Telescopic Tubes, and Base Support.

## Technical Information of Driving Mechanism

The height-adjustment mechanism of the robot is implemented using a motorised screw jack driven by a three-phase AC induction motor. The screw jack provides vertical linear motion via a travelling nut, while the motor supplies the rotational input. The system is designed for controlled, bidirectional vertical motion (up/down) with end-of-travel protection using limit switches. Table 1 shows the technical specifications for the screw jack and motor.

Table 1: Screw Jack Assembly Specifications

| Component | Parameter | Specification |
| --- | --- | --- |
| Screw Jack | Model | HK2T |
|  | Rated lifting capacity (selected) | 300 kg (design requirement) |
|  | Structure type | Travelling nut type |
|  | Nut material | Copper (brass) nut |
|  | Stroke length | 400 mm |
|  | Gear ratio | 1:10 (selected configuration) |
|  | Hoist speed | ~0.70–0.75 m/min |
|  | Estimated travel time (400 mm) | ~32–34 s |
|  | Installation | Vertical mounting, guided by external wear-pad sleeves |
| Motor | Motor type | Three-phase AC induction motor |
|  | Mounting type | B5 flange mount |
|  | Rated power | 0.37 kW |
|  | Rated speed | 1400 r/min (rpm) |
|  | Rated voltage | 380 V / 220 V (dual voltage) |
|  | Rated current | 1.12 A (380 V, Y) / 1.92 A (220 V, Δ) |
|  | Rated frequency | 50 Hz |
|  | Connection | Y (380 V) / Δ (220 V) |
|  | Protection class | IP54 |
|  | Insulation class | Class F |
|  | Motor mass | 6.9 kg |

## Control System Design

Electrical power is supplied from a standard 230 V single-phase wall outlet. The incoming power is first routed through basic protective devices (MCB/RCD/fuse) and a main switch to ensure overcurrent and fault protection. This should be split into two branches:

- Branch A (high power): VFD/contactor system  screw jack motor

- Branch B (low voltage): 230V  transformer to lower voltages for other robot components

This keeps motor noise from killing the other electrical components.

A Variable Frequency Drive (VFD) is used to:

- Convert single-phase AC input into three-phase AC output,

- Supply the appropriate voltage and frequency required by the motor,

- Enable forward and reverse motor operation for bidirectional screw jack travel.

Although speed control is not required for this application, the VFD functions as a phase converter and motor starter, enabling compatibility between wall-plug power and a three-phase induction motor.

The VFD drives a 0.37 kW, three-phase AC induction motor mounted via a B5 flange directly to the screw jack input. The motor operates at approximately 1400 r/min under rated conditions.

Rotational motion from the motor is transmitted through the screw jack’s internal reduction gearbox (ratio 1:10), converting high-speed, low-torque input into low-speed, high-torque output suitable for vertical lifting.

The screw jack employs a travelling nut mechanism, which translates rotational input into vertical linear motion with a stroke of 400 mm. The travelling nut is mechanically connected to the robot’s upper structure.

To prevent lateral loading on the screw and nut, vertical motion is constrained using a guided column system consisting of wear pads or guide sleeves. These guides absorb lateral forces arising from user interaction (e.g., punching impacts), ensuring that the screw jack primarily experiences axial loads.

User Interface

Bidirectional motion (UP/DOWN) is commanded electrically via either:

- A reversing cam switch (UP–OFF–DOWN), or

- Two momentary push buttons (UP / DOWN).

End-of-travel Protection

Two limit switches are installed at the upper and lower ends of travel:

- The top limit switch interrupts upward motion at maximum height,

- The bottom limit switch interrupts downward motion at minimum height.

These limit switches are wired into the control circuit to provide automatic stopping and prevent mechanical over-travel, ensuring safe operation of the height-adjustment mechanism.

The height-adjustment actuator is controlled using a single-phase-input variable frequency drive (VFD) configured for external terminal control. Forward and reverse motion are commanded via dedicated digital inputs (M1 and M2) using momentary UP and DOWN pushbuttons. End-of-travel protection is implemented using upper and lower limit switches wired in series with the corresponding direction command. Although normally open limit switches are used in the current prototype, the control logic ensures that motion is inhibited at the travel limits while still allowing reversal away from the end position. The VFD eliminates the need for external reversing contactors and provides integrated motor protection and soft-start capability.